TYPICAL SPECIFICATION

Armstrong Series BFP Dual Check Backflow Preventers

Furnish and install, as shown on plans and in accordance with manufacturer’s installation instructions. The device shall have two independently acting check valves separated by an intermediate chamber with an atmospheric vent. The device shall feature an integral stainless steel strainer and corrosion resistant internal parts. The device shall be ASSE approved under Standard 1012. The device shall be Armstrong Series BFP.